Miniproject: Brief Report

(ECE 4039- Motion and Geometry Based Methods in Computer Vision)

Name:  Shreya K.R

Registration Numbers: 140907759

Title: Child Safety Monitoring System

Objective:  In the video used, a person is climbing on to certain objects and climbing down. This animated person represents a child in a real life situation. A yellow circle will track the person. The frame numbers, wherever the person climbs above a fixed vertical threshold, will be returned.

Dataset (video) used:

The video of the animated person climbing up onto and down from objects was downloaded from the CMU Graphics Lab Motion Capture Database. The video was found in sub-category Locomotion and was titled ‘Walk on uneven terrain.’ The rolling ball video used to initially test the mean shift tracker was downloaded from YouTube and cropped.

Approach:

Fig: Mean Shift Tracking Algorithm

For each frame of the video

- Q is the pdf of an elliptical region, containing the object to be tracked, in the first frame.

- P is the pdf of consequent rectangular regions in the subsequent frames, which have to be matched with the original pdf q for tracking.

- W is the weight assigned to each pixel in a rectangular region, while calculating the weighted mean.

- The distance d is the inverse of the Bhattacharya coefficient between the pdf’s p and q. The lower the similarity, the higher the distance. The value of the Bhattacharya coefficient should be at least 0.9 for two regions to be considered a match.

EQUATIONS USED and FUNCTIONS DEVELOPED:

Finally cv1.m is the main function, and trackingrect.m function displays a circular tracking window around the object as it moves.

- 6.  To find the new region center (performed by meanshift.m function)

- 4. To find the similarity between the pdf’s of two regions. Max value is 1. (Performed by bcharya.m function)

- 2. Equation to calculate object pdf in subsequent frames (by quantpdf.m function and epkernel.m function)

- 1. Equation to calculate object pdf in frame 1 (performed by initial1.m function, quantpdf.m function and epkernel.m function)

- 5. To find the mean shift vector (performed by meanshift.m function)

- 3. To find the weight corresponding to each pixel  (performed by meanshift.m function)

Output:

The developed mean shift tracker was first tested with a simple animation of a ball rolling down a hill. The results to this can be viewed at: https://www.youtube.com/watch?v=bPns5X9ef0M

The mean shift tracker was then tried on a video of a person climbing onto higher surfaces and getting down again and again. This symbolizes a child climbing on to higher surfaces. The frame numbers every time the person crossed a set vertical threshold were returned.

The results for the video of the walk in irregular terrain can be viewed at: https://www.youtube.com/watch?v=2yAoOCdNFE4

The screenshots for tracking the rolling ball are as follows:

Output:

Initially, the user selects an elliptical region, with the ball in it. A mask is created to select the region of interest.

A yellow circle tracks the motion of the ball in the images, above and below sequentially.

The screenshots for the results of the walk on the unregulated terrain are as follows:

Output:

Initially, the user selects an elliptical region, with the person in it. A mask is created to select the region of interest.

A yellow circle tracks the motion of the person in the images, above and below sequentially. The frame numbers                        corresponding to times when he crosses a physical threshold are returned.

References:

[1]  Comaniciu, Dorin, Visvanathan Ramesh, and Peter Meer. "Kernel-based object tracking." IEEE Transactions on pattern analysis and machine intelligence 25.5 (2003): 564-577.

[2] http://crcv.ucf.edu/courses/CAP5415/Fall2012/Lecture-11-MeanShiftTracking.pdf

[3] http://efavdb.com/mean-shift/